SIMULATION EXPERIMENT 16

SAMPLE CODE:

clc;

clear;

close all;

% Parameters

A = 1;                  % Signal amplitude

f = 5;                  % Signal frequency (Hz)

fs = 1000;               % Sampling frequency (Hz)

t = 0:1/fs:1;

n = 3;                  % Number of quantization bits

L = 2^n;                % Number of quantization levels

% 1. Generate Input Signal

x = A*sin(2*pi*f*t);

% 2. Uniform Quantization

xmin = -A;

xmax = A;

delta = (xmax-xmin)/L;

x_uniform = xmin + delta/2 + ...

delta.*floor((x-xmin)./delta);

x_uniform = min(max(x_uniform,xmin+delta/2), ...

xmax-delta/2);

% 3. Non-Uniform Quantization using Mu-law

mu = 255;

% Compress the input signal

x_compressed = sign(x).* ...

log(1+mu.*abs(x)./A)./log(1+mu);

% Uniform quantization of compressed signal

delta_c = 2/L;

q_compressed = -1 + delta_c/2 + ...

delta_c.*floor((x_compressed+1)./delta_c);

q_compressed = min(max(q_compressed, ...

-1+delta_c/2),1-delta_c/2);

% Expand the quantized signal

x_nonuniform = sign(q_compressed) .* ...

(A .* ((1+mu).^abs(q_compressed)-1) ./ mu);

% 4. Calculate Quantization Errors

error_uniform = x - x_uniform;

error_nonuniform = x - x_nonuniform;

% 5. Calculate Mean Squared Errors

MSE_uniform = mean(error_uniform.^2);

MSE_nonuniform = mean(error_nonuniform.^2);

% 6. Display Results

fprintf('Number of Bits = %d\n',n);

fprintf('Quantization Levels = %d\n',L);

fprintf('Uniform Quantization MSE = %.6f\n',MSE_uniform);

fprintf('Non-Uniform Quantization MSE = %.6f\n',MSE_nonuniform);

% 7. Plot Results

figure('Name','Uniform and Non-Uniform Quantization');

subplot(4,1,1);

plot(t,x,'b','LineWidth',1.2);

title('Original Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(4,1,2);

stairs(t,x_uniform,'r','LineWidth',1);

hold on;

plot(t,x,'b');

title('Uniform Quantization');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized','Original');

grid on;

subplot(4,1,3);

stairs(t,x_nonuniform,'m','LineWidth',1);

hold on;

plot(t,x,'b');

title('Non-Uniform Quantization using Mu-law');

xlabel('Time (s)');

ylabel('Amplitude');

legend('Quantized','Original');

grid on;

subplot(4,1,4);

plot(t,error_uniform,'r');

hold on;

plot(t,error_nonuniform,'m');

title('Quantization Error Comparison');

xlabel('Time (s)');

ylabel('Error');

legend('Uniform','Non-Uniform');

grid on;

sgtitle('Analysis of Uniform and Non-Uniform Quantization');

OUTPUT: