SIMULATION EXPERIMENT 17

SAMPLE CODE:

clc;

clear;

close all;

% Parameters

N = 1000;              % Number of bits

sps = 100;             % Samples per bit

SNR_dB = 10;            % Signal-to-noise ratio in dB

% 1. Generate Random Binary Data

bits = randi([0 1],1,N);

% 2. BPSK Mapping

symbols = 2*bits - 1;

% 3. Generate NRZ Signal

tx = repelem(symbols,sps);

% 4. Add AWGN without Communications Toolbox

signal_power = mean(tx.^2);

noise_power = signal_power/(10^(SNR_dB/10));

noise = sqrt(noise_power)*randn(size(tx));

rx = tx + noise;

% 5. Eye Diagram Plotting

% Each trace contains two bit periods

samples_per_trace = 2*sps;

num_traces = floor(length(tx)/samples_per_trace);

% Time axis for two bit periods

time_axis = (0:samples_per_trace-1)/sps;

% Plot transmitted eye diagram

figure('Name','Digital Communication Eye Diagram');

subplot(2,1,1);

hold on;

for k = 1:min(num_traces,100)

idx = (k-1)*samples_per_trace + (1:samples_per_trace);

plot(time_axis,tx(idx),'b');

end

title('Eye Diagram of Transmitted BPSK Signal');

xlabel('Time (Bit Periods)');

ylabel('Amplitude');

grid on;

xlim([0 2]);

ylim([-1.5 1.5]);

% Plot received eye diagram under AWGN

subplot(2,1,2);

hold on;

for k = 1:min(num_traces,100)

idx = (k-1)*samples_per_trace + (1:samples_per_trace);

plot(time_axis,rx(idx),'r');

end

title(['Eye Diagram of Received Signal with AWGN (SNR = ', ...

num2str(SNR_dB),' dB)']);

xlabel('Time (Bit Periods)');

ylabel('Amplitude');

grid on;

xlim([0 2]);

ylim([-2.5 2.5]);

sgtitle('Eye Diagram Analysis of Digital Communication System');

% 6. Calculate Bit Error Rate

rx_samples = rx(round(sps/2):sps:end);

detected_bits = rx_samples > 0;

BER = sum(bits ~= detected_bits)/N;

fprintf('Input SNR = %.2f dB\n',SNR_dB);

fprintf('Total Bits = %d\n',N);

fprintf('Bit Errors = %d\n',sum(bits ~= detected_bits));

fprintf('Bit Error Rate (BER) = %.6f\n',BER);

OUTPUT: