SIMULATION EXPERIMENT 4

SAMPLE CODE:

clc;

clear;

close all;

% Parameters

fm = 500;               % Message frequency (Hz)

fc = 10000;             % AM carrier frequency (Hz)

fIF = 2000;              % Intermediate frequency (Hz)

fs = 200000;             % Sampling frequency (Hz)

Am = 1;                  % Message amplitude

Ac = 1;                  % Carrier amplitude

mu = 0.7;                % Modulation index

t = 0:1/fs:0.02;

% 1. Message Signal

m = Am*cos(2*pi*fm*t);

% 2. AM Modulated Signal

am = Ac*(1 + mu*m).*cos(2*pi*fc*t);

% 3. Local Oscillator

fLO = fc - fIF;

lo = cos(2*pi*fLO*t);

% 4. Frequency Mixing

mixer = am .* lo;

% 5. Intermediate Frequency (IF) Filtering

[bIF,aIF] = butter(5,[2*(fIF-1000)/fs 2*(fIF+1000)/fs],'bandpass');

if_signal = filtfilt(bIF,aIF,mixer);

% 6. AM Detection using Envelope Detection

envelope = abs(hilbert(if_signal));

% 7. Low-Pass Filtering

[bLP,aLP] = butter(5,2*fm/fs);

recovered = filtfilt(bLP,aLP,envelope);

% Remove DC component and normalize

recovered = recovered - mean(recovered);

recovered = recovered/max(abs(recovered))

% Plot waveforms

figure('Name','Superheterodyne AM Receiver');

subplot(6,1,1);

plot(t,m,'b');

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(6,1,2);

plot(t,am,'b');

title('AM Modulated Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(6,1,3);

plot(t,lo,'b');

title('Local Oscillator Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(6,1,4);

plot(t,mixer,'b');

title('Frequency Mixer Output');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(6,1,5);

plot(t,if_signal,'b');

title('Intermediate Frequency (IF) Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

subplot(6,1,6);

plot(t,recovered,'r','LineWidth',1);

title('Recovered Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

grid on;

sgtitle('Superheterodyne Receiver for AM Communication System');

OUTPUT: